4.  UNITS  OF  ELECTROMAGNETIC  QUANTITIES

1. The measurement of electromagnetic quantities supposes operating with appropriately defined units, which have to supplement in a natural way the set of previously defined measuring units used in mechanics and thermodynamics. The reasonable use of units supposes that these constitute a coherent system of units, which means that consideration is given to the relationships between units in accordance with the relationships between the measured quantities. This way, the units of most electromagnetic quantities can be defined in function of previously defined units, thus reducing the number of so called fundamental units, whose etalon has to be defined on an experimental basis. It can be shown that a single fundamental unit is sufficient to be added to fundamental mechanical and thermal units (m , Kg , s , °K)  in view to define subsequently the units of all electro-magnetic quantities.

The fundamental electromagnetic unit established by the International System of units (IS) is the unit of intensity of the electric conduction current – the ampere. This unit is defined on the basis of Ampere's (electrodynamic) force, acting between two filamentary straight parallel conductors of infinite length, carrying electric conduction currents (fig. 4.1). Such conductors attract each other when the currents flow in the same direction, and repel each other when the currents flow in opposite directions, with an electrodynamic force given by

F=μ i1 i2 l2 π d

Fig. 4.1.

acting on any segment of length  l  of the conductors, where  µ  and  d  are the permeability of the medium and the distance between the filamentary conductors, and  i1  and  i2  the currents carried by them.

An Ampere is the intensity of an electric conduction current which, when carried by each of two filamentary straight parallel conductors of infinite length, placed in free space at 1 meter distance, determines an electrodynamic force of the value

F/l = 2×10–7 Newton/meter

per unit length of each conductor,

áiñ = 1 A  .

2. Starting from this fundamental electromagnetic unit, new units are defined for all other electromagnetic quantities within a coherent system, by making use of the relationships between these quantities.

From equation  i = a + G uC  it follows that the impressed current  a  has the same unit as  i ,  áañ = 1 A  .

From equation  iΣ+iC Σ=−∂qDΣ∂t  it follows that the unit of the convection current  iC  is the same as that of  i , áiCñ = 1 A  .

From equations  iS=J⋅n dS  and  iC=JS×nS⋅dr , it follows the units of current densities  J  and   {JS  as  áJñ = 1A/m2  and  áJSñ = 1A/m , respectively.

From the definition  aS=Ji⋅n dS  it follows that the unit of impressed current density  Ji  is  áJiñ = 1A/m2 .

From the definition  iC=ρVv⋅n dS=JC⋅ndS  it follows that the unit of convection current density  JC  is  áJCñ = 1A/m2 .

From equation  ummΓ=iSΓ+dΨSΓdt  it follows that the magnetic voltage  um  has the same unit as  i ,  áumñ = 1 A  .

From the definition  um=H⋅dr  it follows that the unit of magnetic field strength  H  is  áHñ = 1 A/m  .

From equation  B=μ0H+M  it follows that the magnetisation  M  has the same unit as  H ,  áMñ = 1 A/m  .

From the definition  M=dmdV  it follows that the unit of magnetic moment  m  is  ámñ = 1 A×m2  .

From equation  iΣ=−dqDΣdt  it follows that the unit of electric charge  q  is  áqñ = 1A×1s = 1 C , called  Coulomb .

From definitions  ρV=dqdV  ,  ρS=dqdS  ,  ρL=dqdr , it follows the units of charge densities  V , S , L  as  árVñ = 1 C/m3 , árSñ = 1 C/m2 , árLñ = 1 C/m , respectively.

From equation  ΨΣ=qDΣ  it follows that the electric flux     has the same unit as   q ,  áñ = 1 C  .

From the definition  ΨS=D⋅n dS  it follows the unit of electric displacement  D  as  áDñ = 1 C/m2  .

From equation  D=ε0E+P  it follows that the electric polarisation  P  has the same unit as  D ,  áPñ = 1 C/m2  .

From the definition  P=dpdV  follows the unit of the electric moment  p  as  ápñ = 1 C×m  .

From equation  P = uC i  it follows that the unit of electric voltage  u  is  áuñ = 1W/1A = 1 V , called  Volt .

From equation  uC + e = R i  it follows that the impressed electric voltage  ei  has the same unit as  u ,  áeiñ = 1 V  .

From the definition  u=E⋅dr  it follows that the unit of electric field strength  E  is  áEñ = 1 V/m  .

From the definition  ei=Ei⋅dr  it follows that the unit of the impressed electric field strength  Ei  is  áEiñ = 1 V/m  .

From the relationship between the electric voltage and the electric potential, when the later can be defined,  uAB=VA−VB , it follows that the potential  V  has the same unit as  u ,  áVñ = 1 V  .

From the definition  eΓ=E⋅dr  it follows that the electromotive force  e  has the same unit as  u ,  áeñ = 1 V  .

From equation  eΓ=−dΦSΓdt  it follows that the unit of magnetic flux   is  áñ = 1V×1s = 1 Wb , called  Weber .

From the definition  ΦS=B⋅n dS  it follows the unit of magnetic flux density  B  as  áBñ = 1Wb/1m2 = 1 T , called Tesla .

From equation  uC + e = R i  it follows that the unit of electric resistance  R  is  áRñ = 1V/1A = 1 W , called  Ohm .

From equation   R=ρ lS   it follows that the unit of electric resisitivity     is   árñ  =  1 W×m  .

From equation  i = a + G uC  it follows that the unit of electric conductance  G  is  áGñ = 1A/1V = 1 S , called  Siemens  (or, sometimes,  mho).

From equation   G=σ Sl   it follows the unit of electric conductivity       as   ásñ  =  1 S/m  .

From equation  D=ε0E+P  it follows that the unit of permittivity is given by  ε=1C/m21V/m=1C/1V1m=1Fm , where a new unit was introduced as   1 C1 V=1 F , called  Farad .

From equation  B=μ0H+M  it follows that the unit of permeability is given by  μ=1T1A/m=1Wb/m21A/m=1Wb/1A1m=1Hm , where a new unit was introduced as  1 Wb1 A=1 H  , called  Henry .

Other units for electromagnetic quantities can be introduced according with the definition of such quantities, as it will be seen in chapters  6  and  7  (as, for instance, are the units of electric capacitance or inductance, already used above).